SIMATS ENGINEERING

CO- 2: AT – 2: Simulation Task - Medium

COURSE NAME: Advance Wireless Communication Systems /

5G / 6G Communication Systems

COURSE CODE: ECA56

COURSE OUTCOME COVERED

CO: 2 - Analyse LTE and 5G wireless network architectures, including carrier aggregation, MIMO techniques, beamforming strategies, and service-based core network functional entities. (BL4)

Assessment Tool Weightage: 35%

Each student should simulate one question and should upload the results in Github account along with the necessary outputs and codes. This assessment tool is evaluated for 50 marks.

QUESTIONS:		Total Marks: 50                                 Duration: 90 Mins

- Simulation of LTE throughput performance under varying bandwidth, SNR, modulation order, coding rate, and user load (BL4)

- Simulation of LTE protocol stack delay analysis under uplink traffic, downlink traffic, packet size, retransmission count, and channel quality (BL4)

- Simulation of carrier aggregation performance in LTE-Advanced under single carrier, dual carrier, triple carrier, varying bandwidth, and SNR (BL4)

- Simulation of LTE resource block allocation under different user numbers, scheduling methods, bandwidths, traffic load, and interference levels (BL4)

- Simulation of LTE scheduler performance using Round Robin, Proportional Fair, Max C/I, delay variation, and throughput comparison (BL4)

- Simulation of BER performance in 2×2, 4×4, 8×8 MIMO under varying SNR, modulation schemes, fading conditions, and antenna correlation (BL4)

- Simulation of MIMO channel capacity under antenna number, SNR, bandwidth, fading model, and multiplexing order (BL4)

- Simulation of SU-MIMO performance under channel gain, BER, throughput, latency, and SNR (BL4)

- Simulation of MU-MIMO throughput comparison under user count, antenna count, SNR, modulation, and interference (BL4)

- Simulation of higher-order MIMO diversity gain under fading channel, BER, outage probability, SNR, and antenna configuration (BL4)

- Simulation of beamforming gain under steering angle, antenna elements, frequency, SNR, and interference (BL4)

- Simulation of analog beamforming performance under phase shift variation, beam angle, antenna count, gain, and sidelobe level (BL4)

- Simulation of digital beamforming under user direction, beam width, SINR, antenna spacing, and channel condition (BL4)

- Simulation of hybrid beamforming under RF chains, beam direction, power allocation, SNR, and throughput (BL4)

- Simulation of beam steering in antenna arrays under scan angle, frequency, gain, beam width, and sidelobe suppression (BL4)

- Simulation of 5G throughput performance under QPSK, 16-QAM, 64-QAM, 256-QAM, and varying SNR (BL4)

- Simulation of 5G NR air interface performance under different numerologies, bandwidths, latency, subcarrier spacing, and throughput (BL4)

- Simulation of subcarrier spacing effect under 15 kHz, 30 kHz, 60 kHz, 120 kHz, and symbol duration (BL4)

- Simulation of OFDMA resource allocation under user density, subcarriers, power allocation, throughput, and delay (BL4)

- Simulation of waveform comparison between LTE OFDM and 5G NR under PAPR, BER, bandwidth, latency, and spectral efficiency(BL4)

- Simulation of gNodeB traffic handling under user density, latency, throughput, packet loss, and scheduling (BL4)

- Simulation of functional split in 5G RAN under processing delay, fronthaul load, throughput, latency, and energy consumption (BL4)

- Simulation of RRC procedure timing under handover delay, signaling load, connection setup time, throughput, and failure rate (BL4)

- Simulation of service-based architecture performance under AMF load, SMF load, UPF throughput, latency, and session count (BL4)

- Simulation of network slicing under bandwidth allocation, latency, user priority, throughput, and resource utilization (BL4)

- Simulation of AMF, SMF, and UPF traffic load under session count, packet delay, throughput, CPU utilization, and latency (BL4)

- Simulation of control plane and user plane separation under packet delay, signaling load, throughput, latency, and efficiency (BL4)

- Simulation of 5G core traffic routing under user density, mobility, latency, throughput, and resource utilization (BL4)

- Simulation of packet delivery performance in 5G core under load variation, routing delay, throughput, packet loss, and queue size (BL4)

- Simulation of mobility management under handover count, delay, throughput, signaling load, and failure probability (BL4)

- Simulation of propagation loss under 700 MHz, 3.5 GHz, 28 GHz, 38 GHz, and 60 GHz (BL4)

- Simulation of mmWave path loss under distance, frequency, atmospheric absorption, blockage, and antenna gain (BL4)

- Simulation of channel modeling for mmWave MIMO under LOS, NLOS, fading, delay spread, and SNR (BL4)

- Simulation of interference management under user density, reuse factor, SNR, SINR, and throughput (BL4)

- Simulation of fading channel effects under Rayleigh, Rician, Nakagami, BER, and outage probability(BL4)

- Simulation of V2X communication under latency, vehicle speed, packet delivery ratio, throughput, and interference (BL4)

- Simulation of NB-IoT performance under coverage, delay, power consumption, throughput, and packet loss (BL4)

- Simulation of edge computing delay under local processing, cloud processing, latency, bandwidth, and task size (BL4)

- Simulation of smart city IoT traffic under sensor count, delay, packet delivery, bandwidth, and throughput (BL4)

- Simulation of AI-based wireless optimization under prediction accuracy, throughput, delay, resource allocation, and energy efficiency (BL4)

Code of Conduct:

I ____________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

RUBRICS FOR CALCULATION:

| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Conceptual Understanding Model Design | 20% | Demonstrates strong understanding of underlying concepts in the simulation 5 | Shows good understanding with minor gaps 4 | Partial understanding; some misconceptions 3 | Lacks understanding of key concepts 1-2 |
| Tool Usage | 20% | Uses simulation tools effectively with correct setup and execution 5 | Uses tools with minor errors or guidance 4 | Limited ability to use tools properly 3 | Unable to use simulation tools 1-2 |
| Analysis of Results | 20% | Provides clear, logical analysis and meaningful interpretation of results 5 | Analysis is reasonable with minor gaps 4 | Superficial analysis; limited interpretation 3 | Unable to analyze or interpret results 1-2 |
| Documentation | 20% | Presents results clearly with proper documentation 5 | Documentation is adequate with minor issues 4 | Poorly organized or incomplete documentation 3 | No proper documentation or presentation 1-2 |

| Criteria | Question Number : |  |  |  |
| --- | --- | --- | --- | --- |
|  | Level 4 | Level 3 | Level 2 | Level 1 |
| Conceptual Understanding Model Design |  |  |  |  |
| Tool Usage |  |  |  |  |
| Analysis of Results |  |  |  |  |
| Documentation |  |  |  |  |

CODE :

clc;

clear;

close all;

%% Beam Steering in Antenna Array

% Simulation of beam steering under:

% Scan Angle, Frequency, Gain, Beamwidth and Sidelobe Suppression

%% Basic Parameters

c = 3e8;                  % Speed of light (m/s)

N = 8;                    % Number of antenna elements

fc = 10e9;                % Operating frequency (10 GHz)

lambda = c / fc;          % Wavelength

d = lambda / 2;           % Element spacing

scan_angle = 30;          % Desired beam scan angle (degrees)

theta = -90:0.1:90;       % Observation angle

%% Frequency Variation

frequency = [8e9 10e9 12e9];

%% Element Positions

element_position = (0:N-1) * d;

%% Angle Conversion

theta_rad = deg2rad(theta);

scan_rad = deg2rad(scan_angle);

%% Steering Weights

% Phase shift required for beam steering

phase_shift = -2*pi*d/lambda * sin(scan_rad);

weights = exp(1j * phase_shift * (0:N-1));

%% Array Factor Calculation

AF = zeros(size(theta));

for k = 1:length(theta)

% Phase difference for observation angle

psi = 2*pi*d/lambda * sin(theta_rad(k));

% Array factor

AF(k) = sum(weights .* exp(1j * (0:N-1) * psi));

end

%% Normalize Gain

AF_dB = 20*log10(abs(AF)/max(abs(AF)));

%% Beamwidth Calculation

% Find -3 dB points

indices = find(AF_dB >= -3);

beamwidth = theta(indices(end)) - theta(indices(1));

%% Sidelobe Suppression

% Main beam region

main_beam_region = abs(theta - scan_angle) <= beamwidth/2;

% Find maximum sidelobe level

sidelobe_values = AF_dB(~main_beam_region);

max_sidelobe = max(sidelobe_values);

sidelobe_suppression = -max_sidelobe;

%% Array Gain

array_gain = 10*log10(N);

%% Display Results

fprintf('------------------------------------------\n');

fprintf('      BEAM STEERING SIMULATION\n');

fprintf('------------------------------------------\n');

fprintf('Number of Elements        = %d\n', N);

fprintf('Operating Frequency       = %.2f GHz\n', fc/1e9);

fprintf('Wavelength                = %.4f m\n', lambda);

fprintf('Element Spacing           = %.4f m\n', d);

fprintf('Scan Angle                = %.2f degrees\n', scan_angle);

fprintf('Array Gain                = %.2f dB\n', array_gain);

fprintf('Approximate Beamwidth     = %.2f degrees\n', beamwidth);

fprintf('Sidelobe Suppression      = %.2f dB\n', sidelobe_suppression);

%% Figure 1: Array Geometry

figure('Name','Antenna Array Geometry');

plot(element_position, zeros(size(element_position)), ...

'o', 'LineWidth', 2);

grid on;

xlabel('Position (m)');

ylabel('Amplitude');

title('Uniform Linear Antenna Array');

%% Figure 2: Beam Steering Pattern

figure('Name','Beam Steering Pattern');

plot(theta, AF_dB, 'LineWidth', 2);

hold on;

xline(scan_angle, '--', 'LineWidth', 1.5);

yline(-3, '--', 'LineWidth', 1.2);

grid on;

xlim([-90 90]);

ylim([-50 5]);

xlabel('Angle (Degrees)');

ylabel('Normalized Gain (dB)');

title(['Beam Steering at ', num2str(scan_angle), ' Degrees']);

legend('Array Pattern', ...

'Scan Angle', ...

'-3 dB Level');

%% Figure 3: Polar Radiation Pattern

figure('Name','Polar Radiation Pattern');

polarplot(theta_rad, max(AF_dB, -40), ...

'LineWidth', 2);

title('Polar Radiation Pattern');

%% Figure 4: Frequency Effect

figure('Name','Frequency Effect');

hold on;

for f_index = 1:length(frequency)

current_frequency = frequency(f_index);

current_lambda = c / current_frequency;

current_d = lambda / 2;

current_AF = zeros(size(theta));

% Steering phase is calculated for the original frequency

current_phase_shift = ...

-2*pi*current_d/current_lambda*sin(scan_rad);

current_weights = ...

exp(1j * current_phase_shift * (0:N-1));

for k = 1:length(theta)

psi = 2*pi*current_d/current_lambda ...

* sin(theta_rad(k));

current_AF(k) = sum( ...

current_weights .* ...

exp(1j*(0:N-1)*psi));

end

current_AF_dB = ...

20*log10(abs(current_AF)/max(abs(current_AF)));

plot(theta, current_AF_dB, ...

'LineWidth', 1.5);

end

grid on;

xlim([-90 90]);

ylim([-50 5]);

xlabel('Angle (Degrees)');

ylabel('Normalized Gain (dB)');

title('Effect of Frequency on Beam Steering');

legend('8 GHz', '10 GHz', '12 GHz');

%% Figure 5: Effect of Number of Elements

figure('Name','Effect of Array Size');

hold on;

array_sizes = [4 8 16];

for n_index = 1:length(array_sizes)

current_N = array_sizes(n_index);

current_AF = zeros(size(theta));

current_weights = ...

exp(1j * phase_shift * (0:current_N-1));

for k = 1:length(theta)

psi = 2*pi*d/lambda*sin(theta_rad(k));

current_AF(k) = sum( ...

current_weights .* ...

exp(1j*(0:current_N-1)*psi));

end

current_AF_dB = ...

20*log10(abs(current_AF)/max(abs(current_AF)));

plot(theta, current_AF_dB, ...

'LineWidth', 1.5);

end

grid on;

xlim([-90 90]);

ylim([-50 5]);

xlabel('Angle (Degrees)');

ylabel('Normalized Gain (dB)');

title('Effect of Number of Antenna Elements');

legend('4 Elements', ...

'8 Elements', ...

'16 Elements');

%% Figure 6: Sidelobe Suppression using Amplitude Tapering

figure('Name','Sidelobe Suppression');

% Uniform excitation

uniform_weights = ones(1,N);

% Simple amplitude taper

taper_weights = 0.54 - 0.46*cos(2*pi*(0:N-1)/(N-1));

AF_uniform = zeros(size(theta));

AF_tapered = zeros(size(theta));

for k = 1:length(theta)

psi = 2*pi*d/lambda*sin(theta_rad(k));

AF_uniform(k) = sum( ...

uniform_weights .* weights .* ...

exp(1j*(0:N-1)*psi));

AF_tapered(k) = sum( ...

taper_weights .* weights .* ...

exp(1j*(0:N-1)*psi));

end

AF_uniform_dB = ...

20*log10(abs(AF_uniform)/max(abs(AF_uniform)));

AF_tapered_dB = ...

20*log10(abs(AF_tapered)/max(abs(AF_tapered)));

plot(theta, AF_uniform_dB, ...

'LineWidth', 2);

hold on;

plot(theta, AF_tapered_dB, ...

'LineWidth', 2);

grid on;

xlim([-90 90]);

ylim([-50 5]);

xlabel('Angle (Degrees)');

ylabel('Normalized Gain (dB)');

title('Sidelobe Suppression using Amplitude Tapering');

legend('Uniform Excitation', ...

'Tapered Excitation');

%% Final Message

fprintf('\nSimulation Completed Successfully.\n');

SIMULATION OUTPUT: